ECP Limited

PO Box 34125 Birkenhead, Auckland 0746, New Zealand.

Specification Updated 22.09.2013

Product:-Ethanol Absolute

Catalogue No.22921

| Parameter | Specification |
| --- | --- |
| Assay (ethanol) | 99.5% min |
| Identity (IR) | conforms |
| Colour | <10 Hazen |
| Acidity(as acetic acid) | <.0005 meq/g |
| Alkalinity | <.0002 meq/g |
| Substances reducing KMnO4 | passes test |
| Methanol | 0.01% max |
| Propan-2-ol | 0.002% max |
| Acetone | 0.001% max |
| Acetaldehyde | 0.001% max |
| Water | 0.025% max |
| Residue on Evaporation | 0.001% max |
| Al | 0.00001% max |
| Ba | 0.000002% max |
| Ca | 0.0001% max |
| Cd | 0.000005% max |
| Co | 0.000002% max |
| Cr | 0.000002% max |
| Cu | 0.00001% max |
| Fe | 0.00001% max |
| K | 0.00001% max |
| Mg | 0.00001% max |
| Mn | 0.000002% max |
| Pb | 0.000005% max |
| Zn | 0.00001% max |

Conforms To ACS

Product information.

Formula -C2H5OH; Molecular wt 46.07; Flash point 13 degC ; SG	0.807 at 20 deg C

CAS Number	64-17-5;  HSR No.0001144

2.1GHS Classification Flammable Liquids (Category B) Skin irritation (Category A) Eye irritation (Category A) 2.2 GHS Label elements, including precautionary statements Pictogram Signal word Danger Hazard statement(s) H225 Highly flammable liquid and vapour. H315 Causes skin irritation. H320 Causes eye irritation. Precautionary statement(s) Prevention P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking. P233 Keep container tightly closed. P240 Ground/bond container and receiving equipment. P241 Use explosion-proof electrical/ ventilating/ lighting/ equipment. P242 Use only non-sparking tools. P243 Take precautionary measures against static discharge. P264 Wash skin thoroughly after handling. P280 Wear protective gloves/ protective clothing/ eye protection/ face protection.

Response P303 + P361 + P353 IF ON SKIN (or hair): Remove/ Take off immediately all contaminated clothing. Rinse skin with water/ shower. P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P321 Specific treatment (see supplemental first aid instructions on this label). P332 + P313 If skin irritation occurs: Get medical advice/ attention. P337 + P313 If eye irritation persists: Get medical advice/ attention. P362 Take off contaminated clothing and wash before reuse. P370 + P378 In case of fire: Use dry sand, dry chemical or alcohol-resistant foam for extinction. Storage P403 + P235 Store in a well-ventilated place. Keep cool. Disposal P501 Dispose of contents/ container to an approved waste disposal plant.

Transport and safety data

UN No. 	.	.	1170

DG Class	.	.	3

Packaging Group	.	II

Hazchem code 	.	2[Y]E

Emergency procedure	.	14